Instruction Dependency

And

Pipeline Hazard

S. Ranga Susanth sai

19pw30 MSc SS 3rd Sem

Data Dependency

A Data dependency is a situation in which a program statement (instruction) refers to the data of a preceding statement.

Pipeline Hazards

Hazards are the problems encountered with the instruction pipeline in CPU microarchitecture when the next instruction cannot execute in the following clock cycle.

There are three common types of hazards.

Types of hazards

- Data hazards

- Structural hazards

- Control hazards

Data hazards

Data hazards occur when pipeline changes the order of read/write accesses to operands so that the order differs from the order seen by sequentially executing instructions on the unpipelined machine.

There are three situations in which a data hazard can occur

- Read after Write (RAW) - True dependency

A read after write data hazard refers to a situation where an instruction refers to value which is yet to be computed or retrieved. This can occur because the prior instruction is still being processed while the current instruction requires the result of the previous instruction.

Ex.

Inst1 =>   R1 <-  R2 + R3

Inst2 =>  R5 <- R1 + R3

Here we can see that instruction 2 requires the value stored in register R1 which is still being processed in instruction 1.

- Write after Read (WAR) - Anti dependency

A write after read (WAR) occurs when an instruction tries to write to a register which has not yet been read by a previously issued, but as yet unfinished instruction.

Ex.

Inst1 =>   R1 <-  R2 + R3

Inst2 =>  R3 <- R5 + R4

If the 2nd instruction executes before the 1st instruction it must be ensured that the result of 2nd instruction is not utilized in the 1st instruction.

- Write after Write (WAW) - Output dependence

A write after write hazard occurs when an instruction tries to write its result to the same register as a previously issued, but as a yet unfinished instruction.

Ex.

Inst1 =>   R3 <-  R2 + R4

Inst2 =>  R3 <- R5 + R6

The write back of the 2nd instruction should be delayed until the 1st instruction finishes executing.

Eliminating Data hazards:

There are several solutions used to resolve data hazards

- Insert a pipeline bubble whenever a read after write (RAW) dependency is encountered .

- Use operand forwarding to use data from later stages in the pipeline bubbles.

- Use Out-Of-Order execution to potentially prevent the need of bubbles.

  - Out -Of-Order execution is a paradigm in which a processor executes instructions in an order governed by the availability of input data and execution units, rather than by their original order in a program.

Structural Hazards

Structural hazards occur when two (or more) instructions that are already in the pipeline need the same resource. The result is that instructions must be executed in series rather than parallel for a portion of pipeline. They are also called resource hazards.

Ex.

A machine may have only one register-file write port , but in some cases the pipeline might want to perform two writes in a clock cycle.

Eliminating Structural hazards:

A machine has shared a single-memory pipeline for data and instructions. As a result, when an instruction contains a data-memory reference (load), it will conflict with the instruction references for a later instruction

| Inst\ clockcycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| load | IF | ID | EX | MEM | WB |  |  |  |
| Inst1 |  | IF | ID | EX | MEM | WB |  |  |
| Inst2 |  |  | IF | ID | EX | MEM | WB |  |
| Inst3 |  |  |  | IF | ID | EX | MEM | WB |

To resolve this, we stall the pipeline one clock cycle when a data-memory access occurs. The effect of the stall is actually to occupy the resources for that instruction slot.

| Inst\ clockcycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| load | IF | ID | EX | MEM | WB |  |  |  |  |
| Inst1 |  | IF | ID | EX | MEM | WB |  |  |  |
| Inst2 |  |  | IF | ID | EX | MEM | WB |  |  |
| Stall |  |  |  | bubble | bubble | bubble | bubble | bubble |  |
| Inst3 |  |  |  |  | IF | ID | EX | MEM | WB |

Control hazards

Control hazards occur when the pipeline makes wrong decisions on branch prediction and therefore brings instructions into the pipeline that must subsequently be discarded. The term branch hazard also refers to a control hazard.

Eliminating Control hazards:

The simplest method of dealing with branches is to stall the pipeline as soon as the branch is detected until we reach the MEM stage, which determines the new PC. The pipeline behavior looks like

| Inst\ clock cycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Branch | IF | ID | EX | MEM | WB |  |  |  |  |  |
| Branch successor |  | IF (stall) | stall | stall | IF | ID | EX | MEM | WB |  |
| Branch successor+1 |  |  |  |  |  | IF | ID | EX | MEM | WB |